State Specific Note & Background Note

I.	Introduction

(Why is this topic important for the States/ UTs ? Highlight the data evidence to support the same)

| Providing safe and adequate drinking water through tap connection to every rural household is a key commitment under the Jal Jeevan Mission (JJM). In Madhya Pradesh, MP Jal Nigam Maryadit implements surface-water based multi-village (group) water supply schemes. Each scheme has an intake, water treatment plant, rising mains, overhead tanks and distribution network spread over a large number of villages, and is executed by multiple contractors over several years. |  |
| --- | --- |
| Monitoring such schemes only through progress reports does not show where the work has actually been done, whether it matches the approved DPR, and what assets will be handed over for operation and maintenance. JalRekha has been developed to map every asset of these schemes on GIS and monitor the work in the field through geo-tagged data. |  |
| The following data highlight the scale of the subject: |  |
| Indicator | Status |
| Rural households in the State | 111.89 lakh |
| Rural households with tap connection (as on 03.02.2025) | 75.25 lakh (67.26%) |
| Rural households with tap connection at launch of JJM (15.08.2019) | 13.53 lakh (11%) |
| Multi-village schemes approved in one SLSSC meeting (December 2021) | 22 schemes, ₹15,381.72 crore, 9,240 villages in 15 districts |
| Schemes of MP Jal Nigam monitored on JalRekha | Number of schemes / villages / km of pipeline mapped |
| JalRekha mobile application | 10,000+ installations |
| With investment of this size, spread over thousands of villages, asset-level visibility is necessary both during construction and for long-term operation and maintenance. |  |

II.	Current Situation

(Current policy landscape, programmes, schemes and their progress)

Policy landscape: Under the Jal Jeevan Mission, every rural household is to be provided a functional household tap connection. Surface-water based multivillage schemes are being taken up by MP Jal Nigam, and single-village schemes by the PHE Department. After completion, schemes are to be operated and maintained with the participation of gram panchayats and village water and sanitation committees. At the national level, PM GatiShakti calls for mapping of all utility networks on a common GIS platform.

JalRekha: JalRekha has been developed by MPSeDC for MP Jal Nigam. It consists of a mobile application for field data collection and a web portal for planning, monitoring and reporting. Its main features are:

- Mapping of assets: Pipelines and other components of each scheme are mapped with location, geo-tagged photographs and videos through the mobile application.

- Upload of approved DPR drawings on the portal, so that work done in the field can be compared with the approved design.

- Planned versus actual: Dashboards show planned and actual progress for each scheme.

- Task assignment and monitoring: Activities can be planned and assigned to field teams, and progress monitored in real time.

- Traverse and navigation: Traverse of the water distribution network and GPS navigation to any asset or inspection point.

- Reports and administration: Customised reports, user management and device registration with approval by the administrator.

Technology: .NET Core application with PostgreSQL database and GeoServer

Progress: Number of schemes onboarded; length of pipeline and number of assets mapped; number of field users

III.	Challenges

(Major policy, programmatic and capacity building gaps affecting the working of the States/ UTs Administration)

- Capturing buried assets: Pipelines are underground. If their location is not captured when the trench is open, the as-built network can later be known only from drawings. GPS accuracy of mobile phones is also limited for this purpose.

- Difference between DPR and as-built: Routes and quantities change during execution. Unless every change is captured in the field, the GIS will not match the network actually laid.

- Field data discipline: Data is collected by contractor and field staff with varying skills. Device registration, application updates and data synchronisation in lowconnectivity areas need continuous support.

- Operation and maintenance stage: Once schemes are commissioned, the map has to be used as an asset register for O&M and handed over to gram panchayats. A mechanism for this transition is still to be established.

- Integration with other systems: JalRekha is not yet linked with PM GatiShakti and the other agencies. Pipelines are therefore at risk of damage during road widening and other works.

- Data security: As a system holding critical infrastructure data, encryption of data, access control and security audit need to be strengthened.

- Recurring costs: Hosting, server capacity, licences and support are recurring, while funding is generally available as part of project cost.

IV.	Possible Solutions

(Proposed ideas, actions that can be undertaken by different stakeholders. Scope for convergence with other schemes/ programmes)

- As-built capture as a condition for payment: Geotagged capture of pipeline with the trench open, and of every structure, may be made mandatory before release of the running bill. DGPS devices may be used for higher accuracy on main lines.

- JalRekha as the O and M asset register: After commissioning, the JalRekha map may become the asset register of each scheme, with asset IDs and the record handed over to the gram panchayat.

- Complaints and repairs on the map: Complaints of no supply, leakage and pipe bursts may be recorded with location, and repair work tracked on JalRekha.

- Flow and water quality data: Flow meter readings at key points and water quality test results may be shown on the same map to monitor functionality.

- Strengthening the application: Offline mode, simpler registration, encryption of data on device and in transit, and periodic security audit.

- Capacity building: Training of contractor and field staff in data collection, and of gram panchayat members in using the asset map for O&M.

V.	Best Practices

Madhya Pradesh:

- JalRekha for MP Jal Nigam: A single GIS platform for all schemes of Jal Nigam, with field data collection through mobile application and monitoring through the web portal.

- Planned versus actual monitoring: DPR drawings and field mapped assets on the same map, with dashboards for scheme wise progress.

- Field navigation and traverse: GPS navigation to assets and traverse of the distribution network for inspection.

- Built on State infrastructure: Developed and supported by MPSeDC, and listed on the India Stack directory of NeGD.

Capacity building: Training of field engineers and contractor staff on the JalRekha application. Number of users trained

VI.	Priority Areas

(Selected for implementation by the States in the next 5 years)

| Priority Area | Target (2026–2031) | Lead |
| --- | --- | --- |
| As-built mapping | 100% of Jal Nigam schemes mapped as built, with trench open capture of pipelines | MP Jal Nigam |
| O&M asset register | JalRekha asset register with asset IDs for every commissioned scheme, handed over to gram panchayats | MP Jal Nigam, PHED |
| Functionality monitoring | Flow and water quality data on the map for all multi-village schemes | MP Jal Nigam |
| Complaint redressal | Location-based complaint and repair tracking for all schemes | MP Jal Nigam |
| Integration | APIs with PM GatiShakti and road / power agencies | MPSeDC |
| Security | Encryption, access control and annual security audit | MPSeDC |

VII.	Way Forward – Strategy for Implementation

(Modality for implementation – Legal, Administrative, Technological, Budgetary reforms required for implementation)

Contractual: Inclusion of geo-tagged as built data submission on JalRekha in contract conditions, linked with running bill payments. Notification of JalRekha as the authoritative asset record of MP Jal Nigam, and sharing of utility layers under the PM GatiShakti framework with due security classification.

Administrative: A GIS cell in MP Jal Nigam with MPSeDC support; review of JalRekha progress in scheme review meetings; a defined procedure for handover of the asset map to gram panchayats at the time of scheme handover.

Technological: DGPS or may be RTK for main lines, offline-capable application, , integration of SCADA and water quality data, open APIs, and data encryption and security audit.

Budgetary: A recurring provision for hosting, licences and support within the O&M budget of schemes, and use of Jal Jeevan Mission support funds for IT and monitoring.